## GPS Tracker Communication Protocol

## 1. Summarize

This tracker connects to platform server with TCP. The way for connection is that device connects to the platform server forwardly. After connecting to the platform server, the tracker will feedback a enrolling message. The enrolling message contains the device’s ID. If the device received the answer from the platform server, it will stop to sending enrolling message but send continuous feedback message. The continuous feedback message not contains the device ID. The platform server binds the device by connection. One connection represents a device ID. When the connection cuts off, the device will connect the platform server automatically and send out a device enrolling message. Beside, the device will send out one hand-shaking message intervals of time. The hand-shaking message contains Device ID. After receiving the handshaking answer message from the platform server, the device waits for sending the handshaking message in next period.

## Updated Version Instruction

## 2. Message Instrument

## 2.1 Data Type definition

| Data Type | Instruction |
| --- | --- |
| CHAR | Single ASCII code character |
| C_STRING | Contain ASCII character string. When fix digits, fill in Binary system of bank（0x20H）on right for lacking digit to fix a long time except for special instruction. |
| N_STRING | Contain the digit character string of 0.9. When fix digits, fill in ASCII code 0(Ox30H) on left for lacking digit except for special instruction. |
| H_STRING | Contain the digit character string of O. F. When fix digits, fill in ASCII code 0(Ox30H) on left for lacking digit except for special instruction. |
| HEX_STRING | Hexadecimal system character string. Such as 1, use “31” for indication. When fix digits, fill in ASCII code 0 (Ox30H) on left for lacking digit except for special instruction. |
| BIN | Binary system data |
| BYTE | 8 digits without symbol integer,0..255 |

## 2.2 Message format

GPS Tracker exchanges the information with network gateway through data frames transmitting，using TCP protocol. Full data frames structure definition for GPRS is as following:

| Head | Serial number / Time | Command | Message Body | Trail |
| --- | --- | --- | --- | --- |
| 1 byte | 12 byte | 4 byte | N byte (N≤1K) | 1byte |

Each Full data frame must contain: Head symbol, Serial Number/ Time, Command word, Message body, Trail symbol

## 2.3 Message field definition Y

## 2.3.1 Head/Trail symbol digit

Symbol digit figures the beginning and ending of the message frame. 0x28H (character “(” ) as beginning symbol，and 0x29H (character “)” ) as ending symbol.

### 2.3.2 Command word

Length：4 bytes，C_STRING character

Function：Define the type of operated message for data frame transmitting, and figures the function of data. The definition is as following,

Table 2 Message Definition

| Main first types of Message | Second types of Message | Message serial NO. # | Command description | Remark |
| --- | --- | --- | --- | --- |
| A (Down Message) | P | 00 | One time calling message 3.1.5 | Device parameter message |
|  |  | 03 | Read device parameter configuring message |  |
|  |  | 04 | Read device operated status message |  |
|  |  | 05 | Device login response message 3.1.2 |  |
|  |  | 07 | Center No. configuring message |  |
|  |  | 11 | Cell phone NO. configuring message |  |
|  |  | 12 | Setting vehicle high and low limit speed 3.1.8 |  |
|  |  | 15 | Monitor Command |  |
|  |  | 17 | Read device cell phone configuring |  |
|  | Q | 00 | Common Message | General communication message |
|  |  | 01 | Attemper Message |  |
|  |  | 02 | Answer of calling message(Taxi) |  |
|  |  | 03 | Calling Message(Taxi) |  |
|  |  | 04 | Navigation Message |  |
|  | R | 00 | Isochronous for continues feedback configuring 3.1.3 | Vehicle positioning Message Answer message |
|  |  | 01 | Isometry for continues feedback configuring |  |
|  |  | 05 | Set ACC open sending data transmiting intervals 3.1.12 |  |
|  |  | 06 | Set ACC close sending data transmiting intervals 3.1.13 |  |
|  | S | 01 | Answer Alarm Message 3.1.4 | Answer signal |
|  |  | 07 | Answer Message for getting customer successfully (Taxi) |  |
|  | T | 00 | Control the restarted message of the device 3.1.11 | Control signal |
|  | V | 00 | Circuit control signal 3.1.9 |  |
|  |  | 01 | Oil control signal 3.1.10 |  |
|  |  | 02 | One key configuring command |  |
|  |  | 03 | Read one key configuring |  |
|  | X | 00 | Answer currency up explaining result message | Expanding message |
|  |  | 01 | Alarm configuring message |  |
|  |  | 02 | Device Function configuring command |  |
|  |  | 03 | Device mode configured command |  |
|  |  | 04 | Intialized device command |  |
|  |  | 05 | Setting Geo-fence Message 3.1.14 |  |
| B (Up Message) | O | 01 | Alarm message 3.2.4 | Alarm message |
|  |  | 02 | Answer device parameter configured message |  |
|  |  | 03 | Answer device operated status message |  |
|  |  | 04 | Answer calling message 3.2.5 |  |
|  |  | 05 | Anser device login response message 3.2.2 |  |
|  |  | 12 | Answer vehicle high and low speed limit 3.2.8 |  |
|  |  | 07 | Message for getting customer successfully (Taxi) |  |
|  | R | 00 | Isochronous for continues feedback message 3.2.6 | Vehicle positioning message |
|  |  | 01 | Isometry continous feedback message |  |
|  |  | 02 | Continues feedback ending messsage3.2.7 |  |
|  |  | 05 | Answer the Setting ACC open sending data transmiting intervals 3.2.12 |  |
|  |  | 06 | Answer the Setting ACC close sending data transmiting intervals 3.2.13 |  |
|  | S | 04 | Answer attempered Message | Answer message |
|  |  | 05 | Answer reading called configuring number |  |
|  |  | 06 | Answer calle configuring number |  |
|  |  | 08 | Answer setting isochronous feedback message 3.2.3 |  |
|  |  | 09 | Answer setting Isometry feedback message |  |
|  |  | 20 | Answer response calling message (Taxi) |  |
|  |  | 21 | Answer calling message(Taxi) |  |
|  |  | 23 | Answer navigation message |  |
|  | T | 00 | Answer the restarted message of the device 3.2.11 |  |
|  | U | 00 | Answer the Setting Geo-fence Message 3.2.14 |  |
|  | V | 00 | Answer circuit control 3.2.9 | Answer control sign |
|  |  | 01 | Answer oil control 3.2.10 |  |
|  |  | 02 | Answer enquiring of one key setting |  |

Reserved the non- definition message for expanding message in future

The words in red is the functions the device had.

### 2.3.3 Device ID

Length：15 bytes (Fixed); Type:  C_STRING.

Function：This field for fixing the device. Only when the device sends the device login message and handshake message, it will send the device ID, and other message will not send device ID. The platform fixs device by device ID. The usual format for device ID is ”0000” + ”telephone number”. The reference format is : ”000013612345678”

### 2.3.4 Message running NO. / Time

Length：12 bytes (Fixed); Type: C_STRING

When centre need response message, the 12 bytes figures the message running NO. And device’s feedback should have the same running NO. with the sent message by the centr.Other time, the 12 bytes is the time field.

### 2.3.5 Message body

Length：no fixed,<=1024 bytes，also can be blank。

Function：Confirm the server data message under corresponding command.

# 3. Command Message

## 3.1.Down Message（platform server sending）

### 3.1.2 Device login response message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AP05 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING | non |  |
| Message content |  |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AP05） “13612345678” is tracker ID. |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.3 Same time continues feedback configure

| Message Field | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- |
| Beginning identifier | ( | CHAR | 1 |  |
| Running NO./Time |  | C_STRING | 12 |  |
| Command word | AR00 | C_STRING | 4 |  |
| Message Body |  | C_STRING | 8 |  |
| Message Content | AR00XXXXYYZZ AR00：Fixed key words XXXX：Interval for each message of continues feedback. hex。Unit：Second, 4 characters in all, H_STRING. The max is 0xFFFF seconds。When XXXX＝0,the device stops continues feedback. YYZZ：The total time for feedback, 16 advance system. Unit：YY：Hour、ZZ：Minute. 4 characters in all，H_STRING，The max is 0xFFFF，ie:255 hours 255 minutes. When YYZZ＝0, according to the time intervals, continues feedback. When both XXXX and YYZZ are not 0，it figure that feedback according to the time intervals, when it up to the total time, it automaticly stop to feedback |  |  |  |
| Ending identifier | ) | CHAR | 1 |  |

| For example： |  |
| --- | --- |
| (013612345678AR0000140024) Down fixed time to set continues feedback. Feedback GPS data every 20 (16*1 + 4) seconds and feedback 36 (16 * 2 + 4) minutes in all. “13612345678” is tracker ID. |  |
| Response | Device response BS08 |
| Sending mode | Short Message, GPRS |
| Instruction | This message is available to ecolomic device and navigation device. In the mode of SMS to continues feedback, if set time interval is less than the Min time interval (Set by the device manufacturer),it will continues feedback according to the Min time interval, otherwise continues feedback according to the set time. The data mode is the same as the SMS mode. |

### 3.1.4 Answer Alarm Message

| Message Field |  | Message Value |  | Type |  | Length (Character) |  | Instruction |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( |  | CHAR |  | 1 |  |  |  |
| Running NO./Time |  |  |  | C_STRING |  | 12 |  |  |  |
| Command word |  | AS01 |  | C_STRING |  | 4 |  |  |  |
| Message body |  |  |  | C_STRING |  | 1 |  |  |  |
| Message Content |  | AS01X X：The type of alarm for BO01X up alarm message.1character,16 advance system, ASCII character 0：Cut off vehicle oil 1： Happen accident 2：Vehicle rob (SOS help) 3：Vehicle anti-theft alarm 4：Vehicle low speed alarm 5：Vehicle over speed alarm 6. Alarm out of Geo-fence |  |  |  |  |  |  |  |
| Ending identifier |  |  | ) |  | CHAR |  | 1 |  |  |
| For example: |  |  |  |  |  |  |  |  |  |
| (013612345678AS012) Answer the up vehicle rob police, “13612345678” is tracker ID. |  |  |  |  |  |  |  |  |  |
| Response | No need response |  |  |  |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |  |  |  |  |

### 3.1.5 One time enquiry message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AP00 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING | 0 |  |
| Message body |  |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example: |  |  |  |  |  |
| （013612345678AP00） Closed the oil.“13612345678” is tracker ID. |  |  |  |  |  |
| Response | Device response BP04 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.8 Setting vehicle high and low limit speed

| Message Field |  | Message Value | Type | Length (Character) |  | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |  |
| Running NO./Time |  |  | C_STRING | 12 |  |  |
| Command word |  | AP12 | C_STRING | 4 |  |  |
| Message Body |  | Message content | C_STRING |  |  |  |
| Message Content |  | H050L030 |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |  |
| For example： |  |  |  |  |  |  |
| (013612345678AP12 H050L030 ) Setting the up limit speed is50km/h,low limit is 30km/h.When up limit is 000,it figures cancel alarm up limit, and When down limit is 000,it figures cancel alarm down limit. Less 3 digits of the speed, full 0 on left. Alarm refer to 3.2.4。“13612345678” is tracker ID. |  |  |  |  |  |  |
| Response | BP12 |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |  |

### 3.1.9 Circuit control signal

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AV00 | C_STRING | 4 |  |
| Message Body |  | Message content | C_STRING |  |  |
| Message Content |  | “1”or“0”, “1”figures opening circuit,“0”figures closing circuit. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example： |  |  |  |  |  |
| (013612345678AV000) Closed the circuit, “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BV00 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.10 Oil control single

| Message Field |  | Message Value | Type | Length (Character) |  | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |  |
| Running NO./Time |  |  | C_STRING | 12 |  |  |
| Command word |  | AV01 | C_STRING | 4 |  |  |
| Message body |  | Message content | C_STRING |  |  |  |
| Message content |  | “1”or“0”,“1”figures opening oil，“0”figures closing oil。 |  |  |  |  |
| Ending identifer |  | ) | CHAR | 1 |  |  |
| For example： |  |  |  |  |  |  |
| (013612345678AV010) Closed the oil。“13612345678” is tracker ID. |  |  |  |  |  |  |
| Responds： | BV00 |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |  |

### 3.1.11 Control the restarted message of the device

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AT00 | C_STRING | 4 |  |
| Message body |  | Message Content | C_STRING |  |  |
| Message content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AT00) The device restart. “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BT00 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.12 Set ACC open sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AR05 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | AR05XXXX AR05：Fixed keywords XXXX：The time for sending data intervals for the ACC Open, hex. Unit：Second |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AR050014) It sends back intervals 20 seconds when the ACC is opening. “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BR05 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.13 Set ACC close sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AR06 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | AR06XXXX AR06：Fixed keywords XXXX：The time for sending data intervals for the ACC Open, Hex. Unit：Second |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AR06003C) It sends back intervals 20 seconds when the ACC is closing. “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BR06 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.14 Setting Geo-fence Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AX05 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | AX05 N,D, Minlatitude, Maxlatitude, G, Minlongitude, Maxlongitude AX05：Fixed Keywords N：”0” or “1”，”0”, figures cancel Geo-fence, “1”figures sets Geo-fence. If for cancelling the Geo-fence, the back data cannot be sent out. D：Standard for latitude, N, north latitude; S: south latitude. Minlatitude: lower limit for latitude, Format: DDFF.FFF, DD：latitude’s degree (00 ~ 90), FF.FFF：latitude’s cent (00.0000 ~ 59.999)，reserve three digit decimal fraction. Maxlatitude：upper limit for latitude, Format: DDFF.FFF, DD：latitude’s degree (00 ~ 90), FF.FFF：latitude’s cent (00.0000 ~ 59.999)，reserve three digit decimal fraction. G：Standard for longitude, E, east longitude; S: south longitude. W: west longitude Minlongitude: lower limit for longitude, Format: DDDFF.FFF，DDD： Longitude’s degree (000 ~ 180), FF.FFF：longitude’s cent (00.0000 ~ 59.999), reserve three digit decimal fraction. Minlongitude：upper limit for longitude, Format: DDDFF.FFF，DDD： Longitude’s degree (000 ~ 180), FF.FFF：longitude’s cent (00.0000 ~ 59.999), reserve three digit decimal fraction. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AX051, N,2245.318,2246.452,E,11233.232,11355.175) Set Geo-fence., lower limit for latitude is 22 degree 45．318 cent，upper limit for latitude is 22 degree 46.452 cent; lower limit for longitude is 112 degree 33.232 cent，upper limit for longitude is 113 degree 55.175 cent．“13612345678” is tracker ID. |  |  |  |  |  |
| Response | BU00 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

## 3.2.Up message（The device Sending）

### 3.2.2 Login message

| Message Field | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- |
| Beginning identifier | ( | CHAR | 1 |  |
| Running NO./Time |  | C_STRING | 12 |  |
| Command word | BP05 | C_STRING | 4 |  |
| Device ID | Terminal ID | C_STRING | 15 |  |
| Message body |  | C_STRING | 60 |  |
| Message content | 15 terminal ID ＋ GPS data |  |  |  |
| Ending identifier | ) | CHAR | 1 |  |
| Example： |  |  |  |  |
| (013612345678BP05000013612345678080524A2232.9806N11404.9355E000.1101241323.8700000000L000450AC) |  |  |  |  |
| Response： | Centre service response AP05 |  |  |  |
| Instruction： | This message is available to all device |  |  |  |

### 3.2.3 Continuous answer setting isochronous feedback message

| Message Field |  | Message Value | Type | Length (Character) |  | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |  |
| Running NO./Time |  |  | C_STRING | 12 |  |  |
| Command word |  | BS08 | C_STRING | 4 |  |  |
| Message Body |  |  | C_STRING | 8 |  |  |
| Message Content |  | BS08XXXXYYZZ BS08：Fix key words XXXX：interval of time every each return news。Unit：second，total of 4 bytes，H_STRING，up to 65535 seconds。XXXX＝0, stop to return message。 YYZZ：total return time，Unit：YY：Hour、ZZ：Minute。Total of 4 bytes，hexadecimal，up to FFFF，means 255 hours and 255 minutes。When YYZZ＝0,then ceaselessly return according to the interval of time。 When XXXX and YYZZ unequal to，then means ceaselessly return by time interval，stop return until reach the total time。 |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |  |
| Example： |  |  |  |  |  |  |
| （013612345678BS0800050014） Return GPS data every 5 seconds，total of 20 minutes。 |  |  |  |  |  |  |
| Response： | No need to response |  |  |  |  |  |
| Instruction | This message applies to economically terminals and navigational terminals。Ceaselessly return, after the mode of short message. If the interval of set time is less than the interval of minimum time（set by the terminal manufacturers），then the time of ceaselessly return according to the interval of minimum time，if not, then according to the interval of the set time。Data model and short message model are the same. |  |  |  |  |  |

### 3.2.4 Alarm message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BO01 | C_STRING | 4 |  |
| Message Body |  |  | C_STRING | 61 |  |
| Message Content |  | BO01X+GPS data BO01：Fixed keywords X：Specific alarm information code，1 byte，Hexadecimal。 Alarm information： 0：Vehicle power off 1： 2：Vehicle robbery（SOS help） 3：Vehicle anti-theft and alarming 4：Lowerspeed Alert 5：Overspeed Alert 6:Alarm when out of Geo-fence |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| （013612345678BO012061830A2934.0133 N10627.2544E040.0080331309.6200000000L000770AD） Alarm message and vehicle robbery。GPS data acquisition time is March 31,2008，Universal time is 6:18:30。“A” shows the data available，29 degrees,34.0133 minutes north latitude，106 degrees 27.2544 minutes east longitude，speed is 040.0 km/h，the angle is 309.62 degrees, from due north。”L” means the sum of distance, unit is meter，mileage statistic。 |  |  |  |  |  |
| Response： | Centre response AS01 |  |  |  |  |
| Instruction | This message applies to all terminals。Send the information up to 10 times every30 seconds，No longer to send the information after receive the platform response。 |  |  |  |  |

### 3.2.5 Answer Calling Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BP04 | C_STRING | 4 |  |
| Message Body |  |  | C_STRING | Random length |  |
| Message Content |  | BP04＋GPS data BP04：fix Command Word。 |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example |  |  |  |  |  |
| （013612345678BP04080525A2934.0133N 10627.2544E000.0141830309.6200000000L00000023） Up terminal news（center response by one roll call），GPS data acquisition time is May25,2008，Universal time is 14:18:30，”A” shows the data available，29 degrees,34.0133 minutes north latitude，106 degrees 27.2544 minutes east longitude，speed is 0km/h，the angle is 309.62 degrees, from due north.。 |  |  |  |  |  |
| Response | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.6 Isochronous for continues feedback message

| Message Field | Message Value |  | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier | （ |  | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word | BR00 |  | C_STRING | 4 |  |
| Message body |  |  | C_STRING |  |  |
| Message body | BR00＋GPS data |  |  |  |  |
| Message content | ） |  | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example |  |  |  |  |  |
| (013612345678BR00080612A2232.9828N11404.9297E000.0022828000.0000000000L000230AA) |  |  |  |  |  |
| Response |  | No |  |  |  |
| Instruction |  | This message applies to economically terminals and navigational terminals。Continuously return total time and distance，or receive the message of stop continuously return message from the center., then send the ending message to center。 |  |  |  |

### 3.2.7 Continues feedback ending message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR02 | C_STRING | 4 |  |
| Device ID |  |  | C_STRING | Random length |  |
| Message body |  | BR02 ＋ GPS data |  |  |  |
| Message content |  | ) | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction | This message applies to economically terminals and navigational terminals。Continuously return total time and distance，or receive the message of stop continuously return message from the center., then send the ending message to center |  |  |  |  |

### 3.2.8 Setup the speed of the Car

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BP12 | C_STRING | 4 |  |
| Message body |  | Message Content | C_STRING |  |  |
| Message body |  | H0501L030 |  |  |  |
| Message content |  | ) | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example： |  |  |  |  |  |
| (013612345678BP12 H0501L030 ) |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.9 Control circuit

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Serial number/Time |  |  | C_STRING | 12 |  |
| Command Word |  | BV00 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | “1”or“0”,“1” means circuit has been opened，“0” means circuit has been closed |  |  |  |
| Close Identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.10 Control oil

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BV01 | C_STRING | 4 |  |
| Device ID |  | Message content | C_STRING |  |  |
| Message body |  | “1”or“0”,“1”means oil has been opened，“0”means oil has been closed。 |  |  |  |
| Message content |  | ) | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.11 Answer the restarted message of the device

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BT00 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.12 Answer the Setting ACC open sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR05 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.13 Answer the Setting  ACC close sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR06 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.14 Answer the Setting Geo-fence Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | ＢＵ００ | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | ＢＵ００N ＢＵ００：Command N：0 or 1, “0”figures answer the cancelling Geo-fence. “1” figures answer setting Geo-fence. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

# 4. Appendix

# 4.1. The format definition of GPS location message

| Message Field | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- |
| Time | YYMMDD | N_STRING | 6 | Two bytes for each year/month/day |
| The availability of GPS data |  | CHAR | 1 | “A” or “V”. “A” means the availability of GPS data，”V” means the invalidation of GPS data. |
| Latitude |  | N_STRING | 9 | The unit is degree for he front two bytes，from 0～90；the unit is cent for later seven bytes。 |
| Latitude indicator | “N” or “S” | CHAR | 1 | “N” means north latitude，”S” means south latitude |
| Longitude |  | N_STRING | 10 | The unit is degree for he front three bytes, from 0～180；the unit is cent for later seven bytes |
| Longitude indicator | “E” or “W” | CHAR | 1 | “E” means east longitude，”W” means west longitude |
| Speed |  | N_STRING | 5 | The unit is km/h |
| Time | HHMMSS | N_STRING | 6 | Two bytes of the year/month/day |
| Orientation |  | N_STRING | 6 |  |
| IO State | “0” or “1” | N_STRING | 8 | The 8 bytes of IO The first byte representative of the main power switch, "0" means the main power-on, "1", means the main power-off. The second byte on behalf of the ACC (ignition), "0" means ACC off, "1" means ACC on. The Third byte stand for four states, Hex value from 0 to F, binary value from 0000 to 1111, so can stand for four bit states. First bit stand for oil state, ‘0’ stand for supply oil, ‘1’ stand for stop oil. Second bit stand for electricity state, ‘0’ stand for supply electricity, ‘1’ stand for stop electricity. Third bit stand for SOS state, ‘0’ stand no press button, ‘1’ stand for press button. Fourth bit stand for orange wire input state, ‘0’ stand for input low, ‘1’ stand for input high. fourth, fifth reserved. The three bytes of sixth, seventh, eighth stand for voltage value. The unit is 10mV, Hex format. Value from 0 to 960, voltage from 0V to 24V. Such as 5AE is 5*16*16 + A*16 + E =1454 (10mV) = 14.54V |
| Milepost |  | CHAR | 1 | “L” mean Mileage |
| Mile data |  | H_STRING | 8 | Mile data，Unit: Meter The total mileage. The max is 0xFFFFFFFF |

Shenzhen Legevo Technology Co.,LTD

Tel: 86-755-25911595-605

Fax: 86-755-25911595-608

Email:sales@legevo.com

Web:www.legevo.com

Add: Rm 811-815, 523 building, Bagua 2nd Road,Bagualing, Futian Dist,Shen Zhen, Guang Dong ,China