## GPS Tracker Communication Protocol

## 1. Summarize

This tracker connects to platform server with TCP. The way for connection is that device connects to the platform server forwardly. After connecting to the platform server, the tracker will feedback a enrolling message. The enrolling message contains the device’s ID. If the device received the answer from the platform server, it will stop to sending enrolling message but send continuous feedback message. The continuous feedback message not contains the device ID. The platform server binds the device by connection. One connection represents a device ID. When the connection cuts off, the device will connect the platform server automatically and send out a device enrolling message. Beside, the device will send out one hand-shaking message intervals of time. The hand-shaking message contains Device ID. After receiving the handshaking answer message from the platform server, the device waits for sending the handshaking message in next period.

## Updated Version Instruction

| 2012/2/24 | Add RFID message |
| --- | --- |

## 2. Message Instrument

## 2.1 Data Type definition

| Data Type | Instruction |
| --- | --- |
| CHAR | Single ASCII code character |
| C_STRING | Contain ASCII character string. When fix digits, fill in Binary system of bank（0x20H）on right for lacking digit to fix a long time except for special instruction. |
| N_STRING | Contain the digit character string of 0.9. When fix digits, fill in ASCII code 0(Ox30H) on left for lacking digit except for special instruction. |
| H_STRING | Contain the digit character string of O. F. When fix digits, fill in ASCII code 0(Ox30H) on left for lacking digit except for special instruction. |
| HEX_STRING | Hexadecimal system character string. Such as 1, use “31” for indication. When fix digits, fill in ASCII code 0 (Ox30H) on left for lacking digit except for special instruction. |
| BIN | Binary system data |
| BYTE | 8 digits without symbol integer,0..255 |

## 2.2 Message format

GPS Tracker exchanges the information with network gateway through data frames transmitting，using TCP protocol. Full data frames structure definition for GPRS is as following:

| Head | Serial number / Time | Command | Message Body | Trail |
| --- | --- | --- | --- | --- |
| 1 byte | 12 byte | 4 byte | N byte (N≤1K) | 1byte |

Each Full data frame must contain: Head symbol, Serial Number/ Time, Command word, Message body, Trail symbol

## 2.3 Message field definition Y

## 2.3.1 Head/Trail symbol digit

Symbol digit figures the beginning and ending of the message frame. 0x28H (character “(” ) as beginning symbol，and 0x29H (character “)” ) as ending symbol.

### 2.3.2 Command word

Length：4 bytes，C_STRING character

Function：Define the type of operated message for data frame transmitting, and figures the function of data. The definition is as following,

Table 2 Message Definition

| Main first types of Message | Second types of Message | Message serial NO. # | Command description | Remark |
| --- | --- | --- | --- | --- |
| A (Down Message) | D | 01 | Server send packet to device serial port 3.1.26 | Transparent transfer |
|  |  | 02 | Answer device serial port send packet to server 3.1.27 |  |
|  | F | 01 | Answer device send RFID number 3.1.30 |  |
|  | P | 00 | One time calling message 3.1.5 | Device parameter message |
|  |  | 03 |  |  |
|  |  | 04 |  |  |
|  |  | 05 |  |  |
|  |  | 07 |  |  |
|  |  | 11 |  |  |
|  |  | 12 | Setting vehicle high and low limit speed 3.1.8 |  |
|  |  | 15 |  |  |
|  |  | 17 |  |  |
|  | Q | 00 | Server send message to LCD screen 3.1.24 | General communication message |
|  |  | 01 |  |  |
|  |  | 02 |  |  |
|  |  | 03 |  |  |
|  |  | 04 |  |  |
|  | R | 00 | Isochronous for continues feedback configuring 3.1.3 | Vehicle positioning Message Answer message |
|  |  | 01 |  |  |
|  |  | 05 | Set ACC open sending data transmiting intervals 3.1.12 |  |
|  |  | 06 | Set ACC open sending data transmiting intervals 3.1.13 |  |
|  | S | 01 | Answer Alarm Message 3.1.4 | Answer signal |
|  |  | 07 | Answer LCD screen send message to server 3.1.25 |  |
|  | T | 00 | Control the restarted message of the device 3.1.11 | Control signal |
|  | V | 00 | Circuit control signal 3.1.9 |  |
|  |  | 01 | Oil control signal 3.1.10 |  |
|  |  | 02 |  |  |
|  |  | 03 |  |  |
|  | X | 00 |  | Expanding message |
|  |  | 01 |  |  |
|  |  | 02 |  |  |
|  |  | 03 |  |  |
|  |  | 04 |  |  |
|  |  | 05 | Setting Geo-fence Message 3.1.14 |  |
|  | Y | 01 | Send photo message 3.1.28 | Photo message |
|  |  | 02 | Request send photo packet message 3.1.29 |  |
| B (Up Message) | D | 01 | Answer server send packet to device serial port 3.2.26 | Transparent message |
|  |  | 02 | Device send packet to server 3.2.27 |  |
|  | F | 01 | Send RFID number to server 3.2.30 |  |
|  | O | 01 | Alarm message 3.2.4 | Alarm message |
|  | P | 02 |  |  |
|  |  | 03 |  |  |
|  |  | 04 | Answer calling message 3.2.5 |  |
|  |  | 05 | GPS positon message 3.2.2 |  |
|  |  | 12 | Answer vehicle high and low speed limit 3.2.8 |  |
|  |  | 07 |  |  |
|  | R | 00 | Isochronous for continues feedback message 3.2.6 | Vehicle positioning message |
|  |  | 01 |  |  |
|  |  | 02 | Continues feedback ending messsage3.2.7 |  |
|  |  | 04 | LCD screen send message to server 3.2.25 |  |
|  |  | 05 | Answer the Setting ACC open sending data transmiting intervals 3.2.12 |  |
|  |  | 06 | Answer the Setting ACC open sending data transmiting intervals 3.2.13 |  |
|  | S | 04 |  | Answer message |
|  |  | 05 |  |  |
|  |  | 06 |  |  |
|  |  | 08 | Answer setting isochronous feedback message 3.2.3 |  |
|  |  | 09 |  |  |
|  |  | 20 |  |  |
|  |  | 21 |  |  |
|  |  | 23 | Answer server send message to LCD screen 3.2.24 |  |
|  | T | 00 | Answer the restarted message of the device 3.2.11 |  |
|  | U | 00 | Answer the Setting Geo-fence Message 3.2.14 |  |
|  | V | 00 | Answer circuit control 3.2.9 | Answer control sign |
|  |  | 01 | Answer oil control 3.2.10 |  |
|  |  | 02 |  |  |
|  | Y | 01 | Answer photo request 3.2.28 | Photo message |
|  |  | 02 | Send photo packet to server 3.2.29 |  |

Reserved the non- definition message for expanding message in future

The words in red is the functions the device had.

### 2.3.3 Device ID

Length：15 bytes (Fixed); Type:  C_STRING.

Function：This field for fixing the device. Only when the device sends the device login message and handshake message, it will send the device ID, and other message will not send device ID. The platform fixs device by device ID. The usual format for device ID is ”0000” + ”telephone number”. The reference format is : ”000013612345678”

### 2.3.4 Message running NO. / Time

Length：12 bytes (Fixed); Type: C_STRING

When centre need response message, the 12 bytes figures the message running NO. And device’s feedback should have the same running NO. with the sent message by the centr.Other time, the 12 bytes is the time field.

### 2.3.5 Message body

Length：no fixed,<=1024 bytes，also can be blank。

Function：Confirm the server data message under corresponding command.

# 3. Command Message

## 3.1.Down Message（platform server sending）

### 3.1.3 Same time continues feedback configure

| Message Field | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- |
| Beginning identifier | ( | CHAR | 1 |  |
| Running NO./Time |  | C_STRING | 12 |  |
| Command word | AR00 | C_STRING | 4 |  |
| Message Body |  | C_STRING | 8 |  |
| Message Content | AR00XXXXYYZZ AR00：Fixed key words XXXX：Interval for each message of continues feedback. hex。Unit：Second, 4 characters in all, H_STRING. The max is 0xFFFF seconds。When XXXX＝0,the device stops continues feedback. YYZZ：The total time for feedback, 16 advance system. Unit：YY：Hour、ZZ：Minute. 4 characters in all，H_STRING，The max is 0xFFFF，ie:255 hours 255 minutes. When YYZZ＝0, according to the time intervals, continues feedback. When both XXXX and YYZZ are not 0，it figure that feedback according to the time intervals, when it up to the total time, it automaticly stop to feedback |  |  |  |
| Ending identifier | ) | CHAR | 1 |  |

| For example： |  |
| --- | --- |
| (013612345678AR0000140024) Down fixed time to set continues feedback. Feedback GPS data every 20 (16*1 + 4) seconds and feedback 36 (16 * 2 + 4) minutes in all. “13612345678” is tracker ID. |  |
| Response | Device response BS08 |
| Sending mode | Short Message, GPRS |
| Instruction | This message is available to ecolomic device and navigation device. In the mode of SMS to continues feedback, if set time interval is less than the Min time interval (Set by the device manufacturer),it will continues feedback according to the Min time interval, otherwise continues feedback according to the set time. The data mode is the same as the SMS mode. |

### 3.1.4 Answer Alarm Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AS01 | C_STRING | 4 |  |
| Message body |  |  | C_STRING | 1 |  |
| Message Content |  | AS01X X：The type of alarm for BO01X up alarm message.1character,16 advance system, ASCII character 0：Cut off vehicle oil 1： No use 2：Vehicle rob (SOS help) 3：Vehicle anti-theft alarm 4：Vehicle low speed alarm 5：Vehicle over speed alarm 6. Alarm out of Geo-fence |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example: |  |  |  |  |  |
| (013612345678AS012) Answer the up vehicle rob police, “13612345678” is tracker ID. |  |  |  |  |  |
| Response | No need response |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.5 One time enquiry message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AP00 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING | 0 |  |
| Message body |  |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example: |  |  |  |  |  |
| （013612345678AP00） Closed the oil.“13612345678” is tracker ID. |  |  |  |  |  |
| Response | Device response BP04 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.8 Setting vehicle high and low limit speed

| Message Field |  | Message Value | Type | Length (Character) |  | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |  |
| Running NO./Time |  |  | C_STRING | 12 |  |  |
| Command word |  | AP12 | C_STRING | 4 |  |  |
| Message Body |  | Message content | C_STRING |  |  |  |
| Message Content |  | H050L030 |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |  |
| For example： |  |  |  |  |  |  |
| (013612345678AP12 H050L030 ) Setting the up limit speed is50km/h,low limit is 30km/h.When up limit is 000,it figures cancel alarm up limit, and When down limit is 000,it figures cancel alarm down limit. Less 3 digits of the speed, full 0 on left. Alarm refer to 3.2.4。“13612345678” is tracker ID. |  |  |  |  |  |  |
| Response | BP12 |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |  |

### 3.1.9 Circuit control signal

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AV00 | C_STRING | 4 |  |
| Message Body |  | Message content | C_STRING |  |  |
| Message Content |  | “1”or“0”, “1”figures opening circuit,“0”figures closing circuit. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example： |  |  |  |  |  |
| (013612345678AV000) Closed the circuit, “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BV00 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.10 Oil control single

| Message Field |  | Message Value | Type | Length (Character) |  | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |  |
| Running NO./Time |  |  | C_STRING | 12 |  |  |
| Command word |  | AV01 | C_STRING | 4 |  |  |
| Message body |  | Message content | C_STRING |  |  |  |
| Message content |  | “1”or“0”,“1”figures opening oil，“0”figures closing oil。 |  |  |  |  |
| Ending identifer |  | ) | CHAR | 1 |  |  |
| For example： |  |  |  |  |  |  |
| (013612345678AV010) Closed the oil。“13612345678” is tracker ID. |  |  |  |  |  |  |
| Responds： | BV00 |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |  |

### 3.1.11 Control the restarted of the device

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AT00 | C_STRING | 4 |  |
| Message body |  | Message Content | C_STRING |  |  |
| Message content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AT00) The device restart. “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BT00 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.12 Set ACC open sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AR05 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | AR05XXXX AR05：Fixed keywords XXXX：The time for sending data intervals for the ACC Open, hex. Unit：Second |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AR050014) It sends back intervals 20 seconds when the ACC is opening. “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BR05 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.13 Set ACC close sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AR06 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | AR06XXXX AR06：Fixed keywords XXXX：The time for sending data intervals for the ACC Open, Hex. Unit：Second |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AR06003C) It sends back intervals 20 seconds when the ACC is closing. “13612345678” is tracker ID. |  |  |  |  |  |
| Response | BR06 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.14 Setting Geo-fence Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AX05 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | AX05 N,D, Minlatitude, Maxlatitude, G, Minlongitude, Maxlongitude AX05：Fixed Keywords N：”0” or “1”，”0”, figures cancel Geo-fence, “1”figures sets Geo-fence. If for cancelling the Geo-fence, the back data cannot be sent out. D：Standard for latitude, N, north latitude; S: south latitude. Minlatitude: lower limit for latitude, Format: DDFF.FFF, DD：latitude’s degree (00 ~ 90), FF.FFF：latitude’s cent (00.0000 ~ 59.999)，reserve three digit decimal fraction. Maxlatitude：upper limit for latitude, Format: DDFF.FFF, DD：latitude’s degree (00 ~ 90), FF.FFF：latitude’s cent (00.0000 ~ 59.999)，reserve three digit decimal fraction. G：Standard for longitude, E, east longitude; S: south longitude. W: west longitude Minlongitude: lower limit for longitude, Format: DDDFF.FFF，DDD： Longitude’s degree (000 ~ 180), FF.FFF：longitude’s cent (00.0000 ~ 59.999), reserve three digit decimal fraction. Minlongitude：upper limit for longitude, Format: DDDFF.FFF，DDD： Longitude’s degree (000 ~ 180), FF.FFF：longitude’s cent (00.0000 ~ 59.999), reserve three digit decimal fraction. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| (013612345678AX051, N,2245.318,2246.452,E,11233.232,11355.175) Set Geo-fence., lower limit for latitude is 22 degree 45．318 cent，upper limit for latitude is 22 degree 46.452 cent; lower limit for longitude is 112 degree 33.232 cent，upper limit for longitude is 113 degree 55.175 cent．“13612345678” is tracker ID. |  |  |  |  |  |
| Response | BU00 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.1.24 Server send message to LCD screen

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AQ00 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | L + CCC L ：Message content length, one byte, hex format CCC ：Message content, ASCII code。 |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AQ00 0x0B Welcome !!!） 0x0B is length, “Welcome !!!” is message content. |  |  |  |  |  |
| Response | BS23 |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.1.25 Answer LCD screen send message to server

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AS07 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AS07） |  |  |  |  |  |
| Response | no |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.1.26 Server send packet to device serial port

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AD01 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | B+LLL+CCCCC B：one byte, value set to 1。 LLL：content length, three bytes, hex format。 CCCCC：content, length no fix. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AD011005ABCDE） AD01is command，1 is fix value，005stand for content length is 5 bytes，ABCDE is content，device will send content to device serial port。 |  |  |  |  |  |
| Response | BD01 |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.1.27 Answer device serial port send packet to server

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AD02 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | B B：one byte,1 stand for success, 0 stand for failure. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AD021） |  |  |  |  |  |
| Response | no |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.1.28 Send photo request

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AY01 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | N + S + Q N ：Camera number, one byte, default is 0x00,hex format. S ：photo number, one byte, hex format。 Q ：photo quality, one byte, hex format value: 0x01 photo VGA（640x480）（normal A） 0x02 photo VGA（640x480）（good B） 0x03 photo VGA（640x480）（better C） 0x04 photo VGA（640x480）（very good D） 0x05 photo QVGA（320x240）（normal A） 0x06 photo QVGA（320x240）（good B） 0x07 photo QVGA（320x240）（better C） 0x08 photo QVGA（320x240）（very good D） 0x09 photo QQVGA（160x120）（normal A） 0x0A photo QQVGA（160x120）（good B） 0x0B photo QQVGA（160x120）（better C） 0x0C photo QQVGA（160x120）（very good D） |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AY01 0x00 0x00 0x01） Request camera number 0 photo, photo quality is 640 x 480，normal。 |  |  |  |  |  |
| Response | BY01 |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.1.29 request send photo data packet

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AY02 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | N + S + PP N ：camera number, one byte, hex format. S ：photo number, one byte, hex format。 PP ：photo data packet number, two bytes, hex format, value from 0x0001 start. ） |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AY02 0x00 0x00 0x00 0x08） Request packet number 8 resend. |  |  |  |  |  |
| Response | BY02 |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.1.30 Answer device send RFID number

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | AF01 | C_STRING | 4 |  |
| Message body |  | Message content | C_STRING |  |  |
| Message content |  | N N NN ：2 bit ASCII value answer flag, the same with device send flag. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| For example |  |  |  |  |  |
| （013612345678AF0179） “79” is answer flag. |  |  |  |  |  |
| Response | no |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

## 3.2.Up message（The device Sending）

### 3.2.2 GPS position message

| Message Field | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- |
| Beginning identifier | ( | CHAR | 1 |  |
| Running NO./Time |  | C_STRING | 12 |  |
| Command word | BP05 | C_STRING | 4 |  |
| Device ID | Terminal ID | C_STRING | 15 |  |
| Message body |  | C_STRING | 60 |  |
| Message content | 15 terminal ID ＋ GPS data |  |  |  |
| Ending identifier | ) | CHAR | 1 |  |
| Example： |  |  |  |  |
| (013612345678BP05000013612345678080524A2232.9806N11404.9355E000.1101241323.8700000000L000450AC) |  |  |  |  |
| Response： | Centre service response AP05 |  |  |  |
| Instruction： | This message is available to all device |  |  |  |

### 3.2.3 Continuous answer setting isochronous feedback message

| Message Field |  | Message Value | Type | Length (Character) |  | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |  |
| Running NO./Time |  |  | C_STRING | 12 |  |  |
| Command word |  | BS08 | C_STRING | 4 |  |  |
| Message Body |  |  | C_STRING | 8 |  |  |
| Message Content |  | BS08XXXXYYZZ BS08：Fix key words XXXX：interval of time every each return news。Unit：second，total of 4 bytes，H_STRING，up to 65535 seconds。XXXX＝0, stop to return message。 YYZZ：total return time，Unit：YY：Hour、ZZ：Minute。Total of 4 bytes，hexadecimal，up to FFFF，means 255 hours and 255 minutes。When YYZZ＝0,then ceaselessly return according to the interval of time。 When XXXX and YYZZ unequal to，then means ceaselessly return by time interval，stop return until reach the total time。 |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |  |
| Example： |  |  |  |  |  |  |
| （013612345678BS0800050014） Return GPS data every 5 seconds，total of 20 minutes。 |  |  |  |  |  |  |
| Response： | No need to response |  |  |  |  |  |
| Instruction | This message applies to economically terminals and navigational terminals。Ceaselessly return, after the mode of short message. If the interval of set time is less than the interval of minimum time（set by the terminal manufacturers），then the time of ceaselessly return according to the interval of minimum time，if not, then according to the interval of the set time。Data model and short message model are the same. |  |  |  |  |  |

### 3.2.4 Alarm message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BO01 | C_STRING | 4 |  |
| Message Body |  |  | C_STRING | 61 |  |
| Message Content |  | BO01X+GPS data BO01：Fixed keywords X：Specific alarm information code，1 byte，Hexadecimal。 Alarm information： 0：Vehicle power off 1：Accident 2：Vehicle robbery（SOS help） 3：Vehicle anti-theft and alarming 4：Lowerspeed Alert 5：Overspeed Alert 6:Alarm when out of Geo-fence |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| （013612345678BO019061830A2934.0133 N10627.2544E040.0080331309.6200000000L000770AD） Alarm message and vehicle robbery。GPS data acquisition time is March 31,2008，Universal time is 6:18:30。“A” shows the data available，29 degrees,34.0133 minutes north latitude，106 degrees 27.2544 minutes east longitude，speed is 040.0 km/h，the angle is 309.62 degrees, from due north。”L” means the sum of distance, unit is meter，mileage statistic。 |  |  |  |  |  |
| Response： | Centre response AS01 |  |  |  |  |
| Instruction | This message applies to all terminals。Send the information up to 10 times every30 seconds，No longer to send the information after receive the platform response。 |  |  |  |  |

### 3.2.5 Answer Calling Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BP04 | C_STRING | 4 |  |
| Message Body |  |  | C_STRING | Random length |  |
| Message Content |  | BP04＋GPS data BP04：fix Command Word。 |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example |  |  |  |  |  |
| （013612345678BP04080525A2934.0133N 10627.2544E000.0141830309.6200000000L00000023） Up terminal news（center response by one roll call），GPS data acquisition time is May25,2008，Universal time is 14:18:30，”A” shows the data available，29 degrees,34.0133 minutes north latitude，106 degrees 27.2544 minutes east longitude，speed is 0km/h，the angle is 309.62 degrees, from due north.。 |  |  |  |  |  |
| Response | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.6 Isochronous for continues feedback message

| Message Field | Message Value |  | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier | （ |  | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word | BR00 |  | C_STRING | 4 |  |
| Message body |  |  | C_STRING |  |  |
| Message body | BR00＋GPS data |  |  |  |  |
| Message content | ） |  | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example |  |  |  |  |  |
| (013612345678BR00080612A2232.9828N11404.9297E000.0022828000.0000000000L000230AA) |  |  |  |  |  |
| Response |  | No |  |  |  |
| Instruction |  | This message applies to economically terminals and navigational terminals。Continuously return total time and distance，or receive the message of stop continuously return message from the center., then send the ending message to center。 |  |  |  |

### 3.2.7 Continues feedback ending message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR02 | C_STRING | 4 |  |
| Device ID |  |  | C_STRING | Random length |  |
| Message body |  | BR02 ＋ GPS data |  |  |  |
| Message content |  | ) | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction | This message applies to economically terminals and navigational terminals。Continuously return total time and distance，or receive the message of stop continuously return message from the center., then send the ending message to center |  |  |  |  |

### 3.2.8 Setup the speed of the Car

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BP12 | C_STRING | 4 |  |
| Message body |  | Message Content | C_STRING |  |  |
| Message body |  | H0501L030 |  |  |  |
| Message content |  | ) | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example： |  |  |  |  |  |
| (013612345678BP12 H0501L030 ) |  |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.9 Control circuit

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Serial number/Time |  |  | C_STRING | 12 |  |
| Command Word |  | BV00 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | “1”or“0”,“1” means circuit has been opened，“0” means circuit has been closed |  |  |  |
| Close Identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.10 Control oil

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BV01 | C_STRING | 4 |  |
| Device ID |  | Message content | C_STRING |  |  |
| Message body |  | “1”or“0”,“1”means oil has been opened，“0”means oil has been closed。 |  |  |  |
| Message content |  | ) | CHAR | 1 |  |
| Ending identifier |  |  |  |  |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.11 Answer the restarted message of the device

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BT00 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.12 Answer the Setting ACC open sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR05 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.13 Answer the Setting  ACC close sending data intervals

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR06 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | no |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.14 Answer the Setting Geo-fence Message

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | ＢＵ００ | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | ＢＵ００N ＢＵ００：Command N：0 or 1, “0”figures answer the cancelling Geo-fence. “1” figures answer setting Geo-fence. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.24 Answer server send message to LCD screen

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | ＢS23 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  |  |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| （013632782450BS23） |  |  |  |  |  |
| Response： | No |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.25 LCD screen send message to server

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BR04 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | L + CCC L ：content length, one byte, hex format. CCC ：message content, UNICODE code, hex format. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| （013612345678BR04 0x0B 0x6D 0xF1 0x57 0x33 0x6B 0x22 0x8F 0xCE 0x60 0xA8 0x00 0x38） 0x0E is length，“0x6D 0xF1 0x57 0x33 0x6B 0x22 0x8F 0xCE 0x60 0xA8 0x00 0x38”is “深圳欢迎您8” |  |  |  |  |  |
| Response： | AS07 |  |  |  |  |
| Instruction： | This message is available to all device |  |  |  |  |

### 3.2.26 Answer server send packet to device serial port

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BD01 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | B B：one byte, 1 stand for success, 0 stand for failure. |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| （013612345678BD011） |  |  |  |  |  |
| Response： | no |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.2.27 device serial port send packet to server

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BD02 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | B+LLL+CCCCC B：one byte, default is 1. LLL：packet content length, three bytes, hex format. CCCCC：packet content . |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| （013612345678BD021006ABCDEF） BD02is command，1 fix value，006 stand for packet content is 6 bytes，ABCDEF is packet content。 |  |  |  |  |  |
| Response： | AD02 |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.2.28 answer request photo

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BY01 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | N + S + LLL N ：camera number. Hex format. S ：photo number. Hex format. LLL ：photo packet length, three bytes, high byte first。 |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| (013632782450BY01 0x00 0x00 0x00 0x10 0x33） Photo length is 0x00 * 256 * 256 + 0x10 * 256 + 0x33 = 4147 bytes。If length is 0, stand for failure. |  |  |  |  |  |
| Response： | no |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.2.29 send photo packet to server

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BY02 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | N + S + LL+KK+DDD+CC N ：camera number, one byte, hex format. S ：photo number, one byte, hex format。 LL ：photo packet number, two bytes, high first, hex format, from 1 start。 KK ：photo packet content length, two bytes, hex format. DDD：packet content， CC ：check sum value, two bytes, high byte first. Hex format, use add sum mode, from camera number to packet end |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| (013632782450BY02 0x00 0x00 0x00 0xFF 0x01 0x02 0xFF … … 0xD9 0x03 0xD8） Camera number is 0x00，photo number is 0x00，packet number is 0x00FF,packet content length is 0x0102， packet content is 0xFF, … … 0xD9，check sum is 0x03,0xD8。 |  |  |  |  |  |
| Response： | no |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

### 3.2.30 send RFID number to server

| Message Field |  | Message Value | Type | Length (Character) | Instruction |
| --- | --- | --- | --- | --- | --- |
| Beginning identifier |  | ( | CHAR | 1 |  |
| Running NO./Time |  |  | C_STRING | 12 |  |
| Command word |  | BF01 | C_STRING | 4 |  |
| Message Body |  | Message Content | C_STRING |  |  |
| Message Content |  | N N+ SS +,RFID,RFID… NN ：2 bit ASCII value, SS ：two bit ASCII ,stand for with RFID number sum。 RFID: 10 bit ASCII value, |  |  |  |
| Ending identifier |  | ) | CHAR | 1 |  |
| Example： |  |  |  |  |  |
| (013632782450BF01792,0012345622,0098743256） This packet with two RFID, ID1 is 0012345622, ID2 is 0098743256. |  |  |  |  |  |
| Response： | AF01 |  |  |  |  |
| Instruction： | This message is available to part device |  |  |  |  |

# 4. Appendix I

# 4.1. The format definition of GPS location message

# (013612345678BP05000013612345678101130A2234.0270N11405.9126E000.00623168.540000000000L00000D35)

“（”

1byte，stand for packet start

“013612345678”

12 bytes，stand for tracker ID，first byte fix 0，can change 11 bytes。

“BP05”

4 bytes,  stand for command。

“000013612345678”

15 bytes, also is tracker ID, you can use front。

“101130”

6 bytes,  stand for data, 10 year 11 month 30 day, GMT time。

“A”

1 byte, stand for gps data valid,  "V" stand for gps data no valid

“2234.0270”

9 bytes, stand for latitude, format is ddmm.mmmm

“N”

1 byte, stand for north, "S" stand for south。

“11405.9126”

10 bytes, stand for longitude, format is dddmm.mmmm

“E”

1 Byte, stand for east, "W" stand for west

“000.0”

5 bytes, stand for speed, unit is km/h

“062316”

6 bytes, stand for time, 06 hour 23 minutes 16 seconds, GMT time

“8.5400”

6 bytes, stand for direction, from 0 to 360 degree。

“00000000”

The 8 bits of IO

The first bit representative of the main power switch, "0" means the main power-on, "1", means the main power-off.

The second bit on behalf of the ACC (ignition), "0" means ACC off, "1" means ACC on.

The third byte stand for four states, hex format, from 0 to F, binary is from 0000 to 1111，so can stand for four states. First bit stand for oil states, 0 stand for supply oil states, 1 stand for cut oil states. Second bit reserved. Third bit stand for SOS states, 0 stand for press button, stand for open state. Four bit reserved.

The fourth byte stand for four states, hex format, from 0 to F, binary is from 0000 to 1111, so can stand for four states. First bit stand for door state, 0 stand for door close, 1 stand for door open. Second bit stand for high sensor 1 state, 0 stand for input is low or on connect, 1 stand for input high. Third bit stand for high sensor 2, 0 stand for inout low or no connect, 1 stand for input high. Fourth bit stand for high sensor 3 state, 0 stand for input low or no connect, 1 stand for input high.

The fifth byte stand for four states, hex format, from 0 to F, binary is from 0000 to 1111, so can stand for four states. First bit stand for low sensor 1 state, 0 stand for input high or no connect, 1 stand for input low. Second bit stand for low sensor 2 state, 0 stand for input high or no connect, 1 stand for input low. Third bit stand for low sensor 3 state, 0 stand for input high or no connect, 1 stand for input low. Fourth bit reserved.

The three bits of sixth, seventh, eighth stand for voltage value. The unit is 10mV, Hex format. Value from 0 to 960, voltage from 0V to 24V. Such as 5AE is 5*16*16 + A*16 + E =1454 (10mV) = 14.54V

“L”

1 byte, stand for mileage flag。

“00000D35”

8 bytes, stand for mileage value, hex format，00000D35=3381meters。

“）”

1 byte, stand for packet end